AMI0NW3 / AMI0NW4

V1.00  12/6/93

NetWare Device Driver for

Host Adapters from American Megatrends, Inc.

Command Line Options

--------------------

PORT=n

The PORT command line option instructs the current instance of AMI0NWx to

look for a host adapter at port nnn.  One of the following port addresses

should be used in place of 'n': 330, 130, 230, 334, 134, 234.

ASPIONLY=[0][1][2][3][4][5][6][7]

ASPIONLY=ALL

The ASPIONLY command line option instructs the current instance of AMI0NWx

to not report some devices on the SCSI bus to NetWare.  This is useful when

ASPI needs full control over a direct access device (e.g.  hard disk).  The

option has two syntaxes.  In the first syntax, the SCSI bus IDs of devices

to reserve for ASPI are listed after the '='.  (Note: Do not insert any

spaces between IDs.) In the second syntax, the word 'ALL' is used intead of

listing individual SCSI IDs.  Using 'ALL' will cause AMI0NWx to report none

of the devices to NetWare.

LUNCHK=[0][1][2][3][4][5][6][7]

LUNCHK=ALL

The LUNCHK command line option instructs the current instance of AMI0NWx to

scan for LUNs other than LUN 0 on some devices on the SCSI bus.  This is

useful when a device contains more that one LUN.  The option has two

syntaxes.  In the first syntax, the SCSI bus IDs of devices on which to

check for additional LUNs are listed after the '='.  (Note: Do not insert

any spaces between IDs.) In the second syntax, the word 'ALL' is used

intead of listing individual SCSI IDs.  Using 'ALL' will cause AMI0NWx to

check for additional LUNs on all devices.

MAXUNITS=n

The MAXUNITS command line option sets the maximum number of devices/LUNs

that can be handled by AMI0NWx.  Note: This number is the total number of

units for all instances of AMI0NWx and may only be specified when loading

the first instance of AMI0NWx.  The default value for MAXUNITS is 16.

MAXIOS=n

The MAXIOS command line option sets the maximum number of I/O requests that

can be taken at one time by AMI0NWx.  The default value to MAXIOS is 64,

and the maximum allowable value for MAXIOS is 1024.

DOSASPI=YES | NO

If the DOS ASPI driver (ASPIKERN.SYS) is loaded in CONFIG.SYS, then this

option should be set to YES.  Under NetWare 3.12 this option defaults to

NO.  Under NetWare 4.0 this option defaults to YES.  The DOS ASPI driver

should not be used when running NetWare 3.11.

#