BIOS Support

============

The Stallion Technologies ATA driver provides several interfaces for the

serial device user.  This document describes the interfaces available and

how to activate each interface.

The DOS Interface

-----------------

The standard DOS character device driver interface is supported, but only

when the driver is loaded from "config.sys".  For example, a line similar

to this should appear in your config.sys:

DEVICE=c:\stallion\ata\atacdk.exe -b ec8/64-at,d0000,2a0,10,16

This interface provides a special file for each serial port connected to the

machine.  Just like the standard "COM1" and "COM2" style devices.  The names

of the devices take two forms.

1)  If the driver is loaded without the "-a" option the the devices are

named "COMn" where "n" starts after the last native serial port and

increases.  For example, on a system with one native serial port and

a four port Stallion card, the stallion devices would be com2, com3,

com4 and com5.

2)  If the driver is loaded with the "-a" option the devices are named

according to their hardware location.  Each device takes the format

"COMbpnn" where "b" is the board number, "p" is the panel number and

"nn" is a two digit port number starting from zero for each panel.

In the above example the devices would be named com0000, com0001,

com0002 and com0003.

These devices operate like normal character devices under DOS.  For example

printer output may be redirected to an ATA port with the print command, ie.,

print com0001

The INT14 Interface

-------------------

To enable maximum efficency and compatibility, the ATA driver provides

several options for its INT14 interface.  Unlike the DOS Interface, the

INT14 interface is valid whether the device driver is loaded at runtime

or from "config.sys".

The INT14 interfaces offered by the ATA driver are:

Standard bios interface

The Extended bios interface (as used in PS/2 style machines)

Stallion driver enhancements

Fossil driver enhancements

The default INT14 interface provided is the standard bios interface.

The 'SETINT14' command can be used change and display the current INT14

interfaces that the ATA driver is using.  Running the command,

SETINT14

Will display the interface currently in use, the command

SETINT14 -?

Will display the available interfaces.  For example to choose the Fossil

extensions run the command,

SETINT14 -i 4

The interface can also be selected on the command line for the driver with

the "-i" switch. For example, a to configure a board using the Fossil

interface add,

ata.exe -i 4 .....

1) BIOS Interface

Function 0

Set up a ports parameters,

Input

AH:    set to 0

AL:    Broken into the four bit fields shown below

DX:    A Stallion port number (com1 = 0)

Output

AH:    0xFF on error, otherwise line status

AL:    Modem status

bits 7-5: baud rate

000   110 baud

001   150 baud

010   300 baud

011   600 baud

100   1200 baud

101   2400 baud

110   4800 baud

111   9600 baud

bits 4-3: parity

00    none

01    odd

11    even

bit 2: stop bits

0     1 stop bit

1     2 stop bits

bits 1-0: character size

10    7 bits

11    8 bits

Function 1

Output 1 character

Input

AH:    set to 1

AL:    The character

DX:    A Stallion port number (com1 = 0)

Output

AH:    0xFF on error, otherwise line status

AL:    Modem status, see function 3

Function 2

Input 1 character

Input

AH:    set to 2

DX:    A Stallion port number (com1 = 0)

Output

AH:    The line status, see function 3

AL:    The character

NOTE: this function will not return until a character is read.

Function 3

Read line and modem status.

Input

AH:    set to 3

DX:    A Stallion port number (com1 = 0)

Output

AH:    The line status

AL:    The modem status

The line status bits are broken down as:

Bit 0:  Data Ready (Characters ready to be read)

Bit 1:  Overrun error

Bit 2:  Parity error

Bit 3:  Framing error

Bit 4:  Break detected

Bit 5:  Transmission hold register empty (transmit ok)

Bit 6:  Transmission shift register empty (all data drained)

The Modem status bits are broken down as:

Bit 0:  Modem ready to send status change (CTS has changed)

Bit 1:  Modem on status change            (DSR has changed)

Bit 2:  Telephone ringing status change   (RI  has changed)

Bit 3:  Connection to receiver change     (DCD has changed)

Bit 4:  Modem ready to send status        (CTS)

Bit 5:  Modem on status                   (DSR)

Bit 6:  Telephone ringing status          (RI)

Bit 7:  Connection to receiver            (DCD)

1) The Extended BIOS Interface (EBIOS)

Function 0

As per BIOS function 0

Function 1

As per BIOS function 1

Function 2

As per BIOS function 2

Function 3

As per BIOS function 3

Function 4

Port initialisation

Input

AH:    4

AL:    1 is break on, 0 is break off.

BH:    Parity

0 - None

1 - Odd

2 - Even

3 - Mark

4 - Space

BL:    Stop bits

0 - 1 stop bit

1 - If using 6, 7, 8 data bits, stop bits is 2

If using 5 data bits, stop bits is 1.5

CH:    Data bits

0 - 5 bits

1 - 6 bits

2 - 7 bits

3 - 8 bits

CL:    Baud rate

0 -  110    baud

1 -  150    baud

2 -  300    baud

3 -  600    baud

4 -  1200   baud

5 -  2400   baud

6 -  4800   baud

7 -  9600   baud

8 -  19200  baud

9 -  38400  baud

Stallion specific baud settings:

10 - 57600  baud

11 - 76800  baud

12 - 115200 baud

13 - 50     baud

14 - 75     baud

15 - 134    baud

16 - 200    baud

17 - 1800   baud

18 - 145000 baud

19 - 230400 baud

DX:    A Stallion port number (com1 = 0)

Output

AH:    Line status

AL:    Modem status

Function 5

Port control

Input

AH:    5

AL:    0 means read lines, 1 means set lines

BL:    if writing lines, bit 0 is DTR and bit 1 is RTS.

DX:    A Stallion port number (com1 = 0)

Output

AH:    Line status

AL:    Modem status

BL:    if reading lines, bit 0 is DTR and bit 1 is RTS.

1) The Stallion extensions

The Stallion extensions are designed to be compatible with the Digiboard

extensions.  When using the extensions, it is important to know that

Digiboard usually numbers its ports starting at COM5 (int14 port 4).  To

configure the ATA driver to start at COM5 add the command line option

"-s 4" to the driver command line.

Function 0

as per BIOS or EBIOS

Function 1

as per BIOS or EBIOS

Function 2

as per BIOS or EBIOS

Function 3

as per BIOS or EBIOS

Function 4

as per EBIOS

Function 5

as per EBIOS

Function 6, Subfunction 1

Driver Information

Input

AH:    Set to 6

AL:    Set to 1, the subfunction value

DX:    A Stallion port number (com1 = 0) or 0xFFFF

Output

AH:    0xFF on error

AL:    total number of ports

BX:    driver version

CX:    number of boards

DX:    the first port number

Function 6, Subfunction 2

Board Information

Input

AH:    Set to 6

AL:    Set to 2, the subfunction value

DX:    A Stallion port number (com1 = 0) or 0xFFFF

BX:    the board number (starts from 0)

Output

AH:    0xFF on error, other wise IRQ number

AL:    Board type

UNKNOWN         0

STALLION        1

BRUMBY4         2

ONboard2        3

ONboard         4

BRUMBY8         5

BRUMBY16        6

ONboard/E       7

ONboard/32      9

ONboard2/32     10

ONboardRS       11

XTREAM21        15

Easyio          20

ECH             21

ECH(MC)         22

ECP             23

ECP(E)          24

ECP(MC)         25

ECH(PCI)        26

ECH64(PCI)      27

Easyio(PCI)     28

EC/RA(PCI)      29

BX:    Memory segment of the boards DPRAM.

CX:    number of ports on board.

DX:    the  board IO address.

SI:    port number of first port on board.

Function 6, Subfunction 0xFF

Driver Name

Input

AH:    Set to 6

AL:    Set to 0xFF, the subfunction value

DX:    A Stallion port number (com1 = 0) or 0xFFFF

Output

AX:    Driver version

CX:    total number of ports

ES:BX: pointer to 8 byte unterminated Driver Name.

Function 7

Send Break

Input

AH:    Set to 7

AL:    0 indicates default break time.

1 indicates that BX contains the break time.

BX:    the break time in 10 millisecond tics.

DX:    A Stallion port number (com1 = 0)

Output

AH:    0xFF is error, 0 is success

Function 8

Status check/peek

Input

AH:    Set to 8

DX:    A Stallion port number (com1 = 0)

Output

AH:    0xFF is error, line status if success

AL:    next input character

Function 9

Flush RX and TX buffers

Input

AH:    Set to 9

DX:    A Stallion port number (com1 = 0)

Output

AH:    0xFF is error, 0 is success

Function 0xA

Input Buffer Count

Input

AH:    Set to 0xA

DX:    A Stallion port number (com1 = 0)

Output

DH:    0xFF is error, 0 is success

AX:    number of characters in input buffer

ZF:    1 is error indicator

Function 0xB

Drop Handshake lines

Input:

AH:    Set to 0xB

DX:    A Stallion port number (com1 = 0)

Output:

AH:    0xFF is error, 0 is success

Function 0xC

Get Port Parameters

Input:

AH:    Set to 0xC

DX:    A Stallion port number (com1 = 0)

Output:

AH:    0xFF is error, otherwise:

Bit 0 = Transmit Xon/Xoff processing

Bit 1 = Receive Xon/Xoff processing

AL:    Bit 0 = DTR Rx flow control

Bit 1 = RTS Rx flow control

Bit 4 = CTS Tx flow control

Bit 5 = DSR Tx flow control

Bit 7 = DCD Tx flow control

BH:    Parity, 0 = None, 1 = Odd, 2 = Even

BL:    Stop bits, 0 = 1 bit, 1 = 2 bits , 2 = 1.5 bits

CH:    Character Length, 0=5, 1=6, 2=7, 3=8

CL:    Baud Rate:

0      110    baud

1      150    baud

2      300    baud

3      600    baud

4      1200   baud

5      2400   baud

6      4800   baud

7      9600   baud

8      19200  baud

9      38400  baud

10     57600  baud

11     76800  baud

12     115200 baud

13     50     baud

14     75     baud

15     134    baud

16     200    baud

17     1800   baud

Stallion additions:

18     145000 baud

19     230400 baud

Function 0xD

Get pointer to character ready flag

Input:

AH:    Set to 0xD

DX:    A Stallion port number (com1 = 0)

Output:

ES:BX  Segment/Offset of pointer to character ready flag

Function 0xE

Write buffer

Input:

AH:    Set to 0xE

DX:    A Stallion port number (com1 = 0)

CX:    Number of chars in buffer

ES:BX: Segment/offset of buffer to transmit

Output:

AX:    Number of chars transmitted

DH:    0xFF is error indicator

ZF:    1 is error indicator

Function 0xF

Read buffer

Input:

AH:    Set to 0xF

DX:    A Stallion port number (com1 = 0)

CX:    Number of chars to read

ES:BX: Segment/offset of buffer to fill

Output:

AX:    Number of chars read

DH:    0xFF is error indicator

ZF:    1 is error indicator

Function 0x10

Clear Receive buffer

Input:

AH:    Set to 0x10

DX:    A Stallion port number (com1 = 0)

Output:

AH:    0 = no error, 0xFF Error

Function 0x11

Clear Transmit buffer

Input:

AH:    Set to 0x11

DX:    A Stallion port number (com1 = 0)

Output:

AH:    0 = no error, 0xFF Error

Function 0x12

Get Tx buffer space

Input:

AH:    Set to 0x12

DX:    A Stallion port number (com1 = 0)

Output:

AX:    Number of free bytes in Tx Buffer

Function 0x13

Set Handshake Lines

Input:

AH:    Set to 0x13

DX:    A Stallion port number (com1 = 0)

Output:

AH:    0 = no error, 0xFF Error

Function 0x14

Non-destructive Char read

Input:

AH:    Set to 0x14

DX:    A Stallion port number (com1 = 0)

Output:

AH:    line status

AL:    Next character to be read

Function 0x15

Input byte count

Input:

AH:    Set to 0x15

DX:    A Stallion port number (com1 = 0)

Output:

AX:    Number of bytes in the receive buffer

Function 0x17

Pause/Resume Transmit (not implemented yet)

Input:

AH:    Set to 0x17

DX:    A Stallion port number (com1 = 0)

AL:    0 = pause, 1 = resume

Output:

AH:    0 = no error, 0xFF Error

Function 0x18

Pause/Resume Receive (not implemented yet)

Input:

AH:    Set to 0x18

DX:    A Stallion port number (com1 = 0)

AL:    0 = pause, 1 = resume

Output:

AH:    0 = no error, 0xFF Error

Function 0x1B

Get water marks and buffer sizes

Input:

AH:    Set to 0x1B

DX:    A Stallion port number (com1 = 0)

AL:    0 = Transmit low water, 1 = Receive low water,

2 = Receive hi water

Output:

AX:    Water mark

BX:    The size of the buffer for taget buffer

DH:    0xFF on error

ZF:    Set on error

Function 0x1C

Set Water Marks

Input:

AH:    Set to 0x1C

DX:    A Stallion port number (com1 = 0)

AL:    0 = Transmit low water, 1 = Receive low water,

2 = Receive hi water

BX:    Water mark value

Output:

AH:    0 = no error, 0xFF Error

Function 0x1E

Set Flow Control

Input:

AH:    Set to 0x1E

DX:    A Stallion port number (com1 = 0)

BH:    Bit 0 = Transmit XON/XOFF

Bit 1 = Receive XON/XOFF

Bit 2 = Set XON/XOFF chars (CH=XOFF, CL=XON)

BL:    Bit 0 = DTR Receive

Bit 1 = RTS Receive

Bit 4 = CTS transmit

Bit 5 = DSR transmit

Bit 7 = DCD transmit

CH:    optional XOFF char

CL:    optional XON char

Output:

AH:    0 = no error, 0xFF Error

Function 0x20

Enable/Disable timeouts (not implemented)

Input:

AH:    Set to 0x20

DX:    A Stallion port number (com1 = 0)

AL:    0 = Disable, 1 = Enable

Output:

AH:    0 = no error, 0xFF Error

Function 0xF4

EBIOS test

Input:

AH:    Set to 0xF4

DX:    A Stallion port number (com1 = 0)

Output:

AX:    0 = EBIOS enabled, 0xFF disabled

Function 0xFB

EBIOS modem signals

Input:

AH:    Set to 0xF4

DX:    A Stallion port number (com1 = 0)

AL:    Bit 0 = set DTR status (0 = drop, 1 = raise)

Bit 1 = set RTS status (0 = drop, 1 = raise)

Bit 4 = use CTS output flow control

Bit 5 = use DSR output flow control

Bit 6 = use RI output flow control

Bit 7 = use DCD output flow control

Function 0xFC

Read char no wait

Input:

AH:    Set to 0xF4

DX:    A Stallion port number (com1 = 0)

Output:

AH:    Line status

AL:    Char read

Function 0xFD

Buffer count

Input:

AH:    Set to 0xFD

DX:    A Stallion port number (com1 = 0)

AL:    1 = Tx buffer, 2 = Rx buffer

Output:

CX:    Buffer Count

Function 0xFE

Set pacing mode

Input:

AH:    Set to 0xFE

DX:    A Stallion port number (com1 = 0)

AL:    Bit 6 = XON/XOFF receive pacing

Bit 7 = XON/XOFF transmit pacing

Function 0xFF

Set buffered mode (Not implemented)

Input:

AH:    Set to 0xFF

DX:    A Stallion port number (com1 = 0)

AL:    1 = Tx Buffer, 2 = Rx Buffer

CX:    Buffer size, 0 terminates buffering

ES:BX  Pointer to Buffer

4) The Fossil extensions

The ATA driver implements the serial portion of the FOSSIL interface, to

use the ATA driver with a BBS that requires the FOSSIL extensions, load

your normal fossil driver, then load the ATA driver.  The ATA driver will

then take over the serial portion of the FOSSIL interface for its ports.

The ATA driver supports FOSSIL version 5.

Before the Fossil extensions can be used the driver must be configured

to use the Fossil interface.  To do this run the command,

SETINT14 -i 4

or use the driver command line option '-i' as shown in the first section of

this document.

5) Stallion Extensions

The ATA driver supports one of the above extensions as well as a single

special extension for its own private use.

Function 6, Subfunction fe

ioctl to driver

Input

AH:    Set to 6

AL:    Set to 0xFE

DX:    the port number, 0xFFFF is the control device.

CX:    the command

ES:BX: the argument to the command

Output

AX:    return status of the ioctl