EISACFG

Version 1.41

Septemeber 22, 1992

Michael Brundridge

710 Birch Brook Drive

Leander, Texas 78641

Copyright 1991, 1992 Michael Brundridge

All rights reserved

Table of Contents

1. Introduction . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

1.1 Features . . . . . . . . . . . . . . . . . . . . . . . . . .    1

1.2 Terms in this manual . . . . . . . . . . . . . . . . . . . .    1

2. Using shareware  . . . . . . . . . . . . . . . . . . . . . . . . .    2

2.1 Disclaimer - agreement . . . . . . . . . . . . . . . . . . .    2

2.2 Contacting the author  . . . . . . . . . . . . . . . . . . .    2

3. Running EISACFG  . . . . . . . . . . . . . . . . . . . . . . . . .    3

4. Quick-start  . . . . . . . . . . . . . . . . . . . . . . . . . . .    4

5. Questions asked by EISACFG . . . . . . . . . . . . . . . . . . . .    5

5.1 NAME Statement . . . . . . . . . . . . . . . . . . . . . . .    5

5.2 CATEGORY Statement . . . . . . . . . . . . . . . . . . . . .    5

5.4 SLOT Statement . . . . . . . . . . . . . . . . . . . . . . .    6

5.5 AMPERAGE Statement . . . . . . . . . . . . . . . . . . . . .    6

5.6 DMA Statement. . . . . . . . . . . . . . . . . . . . . . . .    7

5.6.1 SHARED Statement  . . . . . . . . . . . . . . . . . . .    7

5.6.2 TRANSFER SIZE Statement . . . . . . . . . . . . . . . .    7

5.6.3 CYCLE TIMING Statement  . . . . . . . . . . . . . . . .    7

5.7 IRQ Statement  . . . . . . . . . . . . . . . . . . . . . . .    8

5.7.1 SHARED Statement  . . . . . . . . . . . . . . . . . . .    8

5.7.2 TRIGGER Statement . . . . . . . . . . . . . . . . . . .    8

5.8 PORT Statement . . . . . . . . . . . . . . . . . . . . . . .    9

5.8.1 ENDING ADDRESS Statement  . . . . . . . . . . . . . . .    9

5.8.2 SHARED Statement. . . . . . . . . . . . . . . . . . . .    9

5.8.3 SIZE Statement. . . . . . . . . . . . . . . . . . . . .    9

5.9 RAM ADDRESS Statement  . . . . . . . . . . . . . . . . . . .   10

5.9.1 MEMORY Statement  . . . . . . . . . . . . . . . . . . .   10

5.9.2 MEMTYPE Statement . . . . . . . . . . . . . . . . . . .   10

5.9.3 SHARED Statement  . . . . . . . . . . . . . . . . . . .   11

5.9.4 CACHED Statement  . . . . . . . . . . . . . . . . . . .   11

5.9.5 TYPE Statement  . . . . . . . . . . . . . . . . . . . .   11

5.9.6 DECODE Statement  . . . . . . . . . . . . . . . . . . .   11

5.10 RAM ADDRESS Statement  . . . . . . . . . . . . . . . . . . .   12

5.10.1 MEMORY Statement  . . . . . . . . . . . . . . . . . . .   12

5.10.2 CACHED Statement  . . . . . . . . . . . . . . . . . . .   12

5.10.3 TYPE Statement  . . . . . . . . . . . . . . . . . . . .   13

5.10.4 DECODE Statement  . . . . . . . . . . . . . . . . . . .   13

6. !ISA0001.CFG Listing . . . . . . . . . . . . . . . . . . . . . . .   14

7. Using !ISA0001.CFG . . . . . . . . . . . . . . . . . . . . . . . .   16

8. Renaming !ISA0001.CFG  . . . . . . . . . . . . . . . . . . . . . .   16

- i -

1. Introduction

EISACFG is an easy to use program.  The main purpose of EISACFG is to

generate GENERIC EISA CONFIGURATION FILES for the Industry Standard

Architecture (ISA) based card being used in an Extended Industry Standard

Architecture (EISA) based system.

EISACFG will write a configuration file based on answers you supplied to

the questions asked by EISACFG.

1.1 Features

o    Simple operation.

o    Two modes, STANDARD and ADVANCED

1.2 Terms in this manual

The following conventions are used in this manual:

o    The Enter or Return key on the keyboard is displayed as <Enter>.

- 01 -

2. Using freeware

Freeware distribution gives users the ability to use the software at no

charge.

Copyright laws apply to both Freeware and commercial software, and the

copyright holder retains all rights, with a few specific exceptions as

stated below.

The author specifically grants the right to copy and distribute the

software, to all groups.

2.1 Disclaimer/Agreement

Users of EISACFG must accept this disclaimer of warranty:

EISACFG--is supplied as is.  The author disclaims all warranties,

expressed or implied, including, without limitation, the warranties of

merchantability and of fitness for any purpose. The author assumes no

liability for damages, direct or consequential, which may result from the

use of EISACFG.

EISACFG is a "freeware program" and is provided at no charge to the user.

Feel free to share it with your friends, but please do not give it away

altered or as part of another system.

There may be no Commercial use or distribution of EISACFG.

If you need or wish to say thanks for this utility in a monetary manner,

you can send $5 to a charitable organization of your choice.

You are encouraged to pass a copy of EISACFG along to your friends for

use.

All trademarks referenced in this document are the property of their

respective owners.

2.2 Contacting the author

There are several ways to contact Michael Brundridge, the author of

EISACFG.

The easiest way is through the Internet--Send a message to mikeb@dell.com

or mikeb@pandora.dell.com.

- or -

By US Postal Mail--write to the address on the first page of this

document.  He can also be contacted through

- 02 -

3. Running EISACFG

The command string for running EISACFG is:

EISACFG [options]

The items in brackets are optional.  The easiest and suggested way to

run EISACFG, from the command line:

C:\>EISACFG<Enter>

(<Enter> means to hit the Enter or Return key on the keyboard.)

Options are single letters preceded by a forward slash (/), or a dash

(-), i.e. -A or /A.

C:\>EISACFG /A<Enter>

EISACFG will display a banner page with version number and a HELP SCREEN

by using the ?. /?, or -? option:

C:\>EISACFG ?<Enter>

- 03 -

4. Quick-start

To get a feel for how to use EISACFG, just type EISACFG at the DOS prompt

and press <Enter>.  It will display the banner page and the first

question.

����������������������������������������������������������������������������Ŀ

�                 Copyright  Michael Brundridge  1991, 1992                  �

�             Generic ISA card EISA CONFIGURATOR - Version 1.41              �

�                           Press ESC to QUIT                                �

������������������������+>RUNNING STANDARD MODE<+�����������������������������

Enter the card manufacturer's name below:

Default: Generic ISA

What is the card manufacturer's name?

- 04 -

5. Questions asked by EISACFG

5.1 NAME Statement:

Enter the card manufacturer's name below:

Default: Generic ISA

What is the card manufacturer's name?

Syntax:

NAME = "descriptive name"

The NAME statement contains text that identifies the product.  Part

numbers and other information may also be included.  The NAME text

field can contain up to 90 ASCII characters.

------------------------------------------------------------------------

5.2 CATEGORY Statement:

Choices:

COM = Communications Device      NPX = Numeric Coprocessor

KEY = Keyboard                   OSE = Operating System/Environment

MEM = Memory Board               OTH = Other

MFC = Multifunction Board        PAR = Parallel Port

MSD = Mass Storage Device        PTR = Pointing Device

NET = Network Board              SYS = System Board

VID = Video Board

Default: OTH

Enter a three digit code for card CATEGORY from above?

Syntax:

CATEGORY = "3-character category"

The CATEGORY statement contains a 3-character ASCII string that

identifies the board's functional category.  The configuration utility

displays the CATEGORY text field during system configuration.

The CATEGORY statement must use one of the categories shown above

- 05 -

5.3 SLOT Statement:

Choices: (Y)es, (N)o

Default: Y

Is the ISA card a 16-bit card?

Syntax:

SLOT = value [,"text"]...

The SLOT statement identifies the type of slot in which the expansion

board can be installed.  Options that can be entered in the value field

include: ISA8, or ISA16

The value entered in the SLOT field is the actual size of the board's

card edge. For example, an expansion board with an 8 bit card edge is

set to SLOT = ISA8, an expansion board with a 16 bit card edge is set

to SLOT = ISA16, and so on.

------------------------------------------------------------------------

5.5 AMPERAGE Statement:

The question below for AMPERAGE is for the +5 Volt line and should be

in milliamps.

We suggest you try to locate this value so that the EISA config can

inform you when you have reached the current limit of the power supply.

Default: 0

+5 Volt AMPERAGE draw in milliamps (i.e. 300)?

Syntax:

[AMPERAGE = value]

The AMPERAGE statement, specifies the maximum amount of continuous 5V

current (in milliamps) required by the base configuration of the

expansion board.

Devices that require +5 volt power should include an AMPERAGE

statement. The configuration utility cannot perform an accurate power

usage verification if expansion boards that do not specify their power

requirement.

- 06 -

5.6 DMA Statement:

(The DMA section specifies the choice of DMA channels, whether the

channel can be shared, the channel's data size, the channel's cycle

timing.)

Enter the DMA from the list below, or press ENTER if not used:

Choices: 1, 2, 3, 4, 5, 6, 7

Default: NONE

What is the DMA Channel?

Syntax:

DMA = value

The DMA statement marks the beginning of a DMA Request Block and

specifies the DMA channel number supported by the configuration.

------------------------------------------------------------------------

5.6.1 DMA (SHARED) Statement:

Choices: (Y)es, (N)o

Default: N

Will the DMA Channel be SHARED?

------------------------------------------------------------------------

5.6.2 DMA (TRANSFER SIZE) Statement: [ADVANCED OPTION]

Choices are:  BYTE, WORD

Default: If DMA = 1-3 'BYTE', if DMA = 4-7 'WORD'

What SIZE DMA transfer is used?

------------------------------------------------------------------------

5.6.3 DMA (CYCLE TIMING) Statement: [ADVANCED OPTION]

Choices are: DEFAULT, TYPEA, TYPEB

Default: DEFAULT

What type of bus cycle (TIMING) is used for DMA transfers?

- 07 -

5.7 IRQ Statement:

Enter the IRQ from the list below, or press ENTER if not used:

Choices: 3, 4, 5, 6, 7, 9, 10, 11, 12, 13, 14, 15

Default: NONE

IRQ being used?

Syntax:

IRQ = Interrupt number

The IRQ statement specifies the interrupt number supported by the

configuration.

Each interrupt selected during system configuration is stored in

nonvolatile memory with the appropriate SHARE and TRIGGER

characteristics, as well as the EDGE/LEVEL interrupt controller

setting.

------------------------------------------------------------------------

5.7.1 IRQ (SHARED) Statement:

Choices: (Y)es, (N)o

Default: N

Will the IRQ be SHARED?

------------------------------------------------------------------------

5.7.2 IRQ (TRIGGER) Statement: [ADVANCED OPTION]

Choices: EDGE, LEVEL

Default: If SHARE = NO 'EDGE', if SHARE = YES 'LEVEL'

How is the INTERRUPT controller TRIGGERED?

- 08 -

5.8 PORT Statement:

Enter the BASE I/O PORT address for the card, or press ENTER if not used:

Choices: 100H-FFFH

Default: NONE

Starting BASE I/O PORT Address (i.e. 300)?

Syntax:

PORT = rangelist

The PORT statement may specify a single address, a range of addresses.

NOTE: The I/O address should be 3 digits in length.  The real first

usable address by an I/O card is 100.

------------------------------------------------------------------------

5.8.1 PORT (ENDING ADDRESS) Statement:

Ending BASE I/O PORT Address (i.e. 30F)?

NOTE: The I/O address should be 3 digits in length.  The real first

usable address by an I/O card is 100.

------------------------------------------------------------------------

5.8.2 PORT (SHARED) Statement:

Choices: (Y)es, (N)o

Default: N

Will the BASE I/O PORT be SHARED?

------------------------------------------------------------------------

5.8.3 PORT (SIZE) Statement: [ADVANCED OPTION]

Choices: BYTE, WORD

Default: BYTE

What SIZE is the PORT?

- 09 -

5.9 ADDRESS Statement: (for RAM)

Enter the RAM location from the possible list below, paying attention to

not cause a memory address conflict, or press ENTER if no RAM present:

0C000 through FF000

Default: NONE

RAM location (i.e. 0C800)?

ADDRESS Statement (Optional)

Syntax:

ADDRESS = list

The ADDRESS statement specifies the starting address of the

READ/WRITABLE memory on the card.

NOTE: Make certain to proceed any address below 1MB with a 0 as shown

in the example above. (i.e. if the address is C800 enter 0C800).

------------------------------------------------------------------------

5.9.1 MEMORY Statement: (for RAM)

Choices: 8K, 16K, 32K, 64K, 128K

Default: 32K

How much memory is located on the card?

Syntax:

MEMORY = length

The length following the MEMORY statement specifies the maximum

size of the memory (in Kilobytes) installed on the board.

------------------------------------------------------------------------

5.9.2 MEMTYPE Statement: (for RAM)

Choices:

SYS = System             EXP = Expanded

VIR = Virtual            OTH = Other

Default: OTH

MEMORY Type from above list?

Syntax:

[MEMTYPE = SYS � EXP � VIR � OTH]

- 10 -

The MEMTYPE field specifies whether the memory is SYStem (base and

extended memory), EXPanded, OTHer (address used for memory mapped I/O

or bank switched memory), or VIRtual (indicates the space is being

used, but no physical memory occupies the address, i.e. address of a

LIM page frame).

------------------------------------------------------------------------

5.9.3 MEMORY (SHARED) Statement: (for RAM)

Choices: (Y)es, (N)o

Default: Y

Will the MEMORY be SHARED?

------------------------------------------------------------------------

5.9.4 MEMORY (CACHED) Statement: (for RAM)

Choices: (Y)es, (N)o

Default: N

Should the RAM be CACHED?

This will set the 80486 CPU up so it will allow the address to be

cached.

This option can cause problems.  If you turn it on and start having

problems with other hardware, or even programs, try turning it OFF and

running it again.

------------------------------------------------------------------------

5.9.5 MEMORY (TYPE) Statement: (for RAM) [ADVANCED OPTION]

Choices: BYTE, WORD

Default: BYTE

What type is the memory?

------------------------------------------------------------------------

5.9.6 MEMORY (DECODE) Statement: (for RAM) [ADVANCED OPTION]

Choices: 20 (Byte), 24 (Word)

Default: 20

How many DECODE lines are on the board?

- 11 -

5.10 ADDRESS Statement: (for ROM)

Enter the ROM location from possible list below, paying attention to

not cause a memory address conflict, or press ENTER if no ROM present:

0C000 through FF000

Default: NONE

ROM location? (i.e. 0D000)?

ADDRESS Statement (Optional)

Syntax:

ADDRESS = list

The ADDRESS statement specifies the starting address of the

READ ONLY memory on the card.

NOTE: Make certain to proceed any address below 1MB with a 0 as shown

in the example above. (i.e. if the address is D000 enter 0D000).

------------------------------------------------------------------------

5.10.1 MEMORY Statement: (for ROM)

Choices:  8K, 16K, 32K, 64K, 128K

Default: 64K

What is the ROM size?

Syntax:

MEMORY = length

The length following the MEMORY statement specifies the maximum

size of the ROM (in Kilobytes) installed on the board.

------------------------------------------------------------------------

5.10.2 MEMORY (CACHED) Statement: (for ROM)

Choices: (Y)es, (N)o

Default: N

Should the ROM be CACHED?

This will set the 80486 CPU up so it will allow the address to be

cached. This option can cause problems.  If you turn it on and start

having problems with other hardware, or even programs, try turning it

OFF and running it again.

- 12 -

5.10.3 MEMORY (TYPE) Statement: (for ROM) [ADVANCED OPTION]

Choices: BYTE, WORD

Default: BYTE

What type is the ROM?

------------------------------------------------------------------------

5.10.4 MEMORY (DECODE) Statement: (for ROM) [ADVANCED OPTION]

Choices: 20 (Byte), 24 (Word)

Default: 20

How many DECODE lines does the ROM use?

- 13 -

6. !ISA0001.CFG

Below is a listing of the file created by EISACFG that will be used by

your system vendor supplied EISA CONFIGURATOR program.

BOARD

ID = "ISA0001"

NAME = "Generic ISA board, Generated with EISACFG v.1.41"

MFR = "TOKENRING (W/RPL ROM)"

CATEGORY = "NET"

SLOT = ISA16

AMPERAGE =  650                        ;Max +5 VOLT current draw

COMMENTS = "\n This is a GENERIC EISA CONFIG file to be used for ISA cards

where no .CFG file has been supplied by the card vendor.\n\n"

GROUP = "Generic ISA card setup"

FUNCTION = "Generic ISA card DMA Channel"

COMMENTS = "Set generic ISA DMA Channel"

CHOICE = "5"

LINK

DMA = 5

SHARE = NO

SIZE = WORD

TIMING = DEFAULT

FUNCTION = "Generic ISA card IRQ setting"

COMMENTS = "Set generic ISA IRQ setting"

CHOICE = "10"

LINK

IRQ = 10

SHARE = NO

TRIGGER = EDGE

FUNCTION = "Generic ISA card PORT Address"

COMMENTS = "Set generic ISA PORT Address"

CHOICE = "A20H-A2FH"

LINK

PORT = A20H-A2FH

SHARE = NO

- 14 -

FUNCTION = "Generic ISA card RAM Address"

COMMENTS = "Set generic ISA RAM ADDRESS"

CHOICE = "0C8000H for 16K"

LINK

MEMORY = 16K

ADDRESS = 0C8000H

MEMTYPE = OTH

WRITABLE = YES

SHARE = YES

CACHE = NO

SIZE = BYTE

DECODE = 20

FUNCTION = "Generic ISA card ROM Address"

COMMENTS = "Set generic ISA ROM ADDRESS"

CHOICE = "0D0000H for 32K"

LINK

MEMORY = 32K

ADDRESS = 0D0000H

MEMTYPE = OTH

WRITABLE = NO

SHARE = NO

CACHE = NO

ENDGROUP

- 15 -

7. Using !ISA0001.CFG

After EISACFG has completed and generated the !ISA0001.CFG file you will

need to place it where it is needed.  Most vendor supplied EISA

CONFIGURATION programs will require the .CFG file to reside on the EISA

CONFIGURATION Utility diskettes itself.

Due to various EISA CONFIGURATION programs and even different version of

the utility and the way they handle the moving/copying of a new .CFG file

to the utility diskette, it is normally easier to just copy it using DOS

directly to the diskette.

8. Renaming !ISA0001.CFG

If you wish to rename the .CFG file you will need to change the name

within the !ISA0001.CFG and the filename also to match each other.

With an ASCII edit, edit the !ISA0001.CFG and edit the line that looks

like below:

ID = "ISA0001"

Edit the [ID = "ISA0001"] line to use the name you have selected, using

the following Guidelines. The seven character ID consists of a three

character manufacturer code, a three character hexadecimal product

identifier, and a one character hexadecimal revision number.  For

example, the forth revision of an expansion board manufactured by the

DIAMOND Board company might have the ID DIA0104.

ID = "DIA0104"

After having made this change, rename the file to equal the name chosen

and entered into the file.  Remember that the filename must be proceeded

with an exclamation mark "!", i.e. !DIA0104.

- 16 -